OrbitSight

Real-Time Detection & Tracking of Resident Space Objects from Neuromorphic Vision Data

Technical Research Report

TII OrbitSight Challenge — Propulsion & Space Research Center

Problem class: Event-based (neuromorphic) object detection & tracking for Space Situational Awareness Core constraint: CPU-only inference, < 40 ms end-to-end latency, offline container Proposed approach: Learned spatiotemporal-coherence event classification + geometric trajectory aggregation Status: Method design grounded in SSA + event-denoising literature; validation experiment specified

Contents

# 1.  Executive Summary

The task. Given raw neuromorphic vision sensor (NVS) event streams captured by event cameras on a 0.8 m telescope, detect and track Resident Space Objects (RSOs) — satellites, rocket bodies, debris, and apparently-moving stars — producing one bounding box per 40 ms time window, scored at IoU ≥ 0.5 and aggregated into mAP, precision, recall and F1. Inference must run on a CPU-only host within a 40 ms end-to-end latency budget, fully offline, inside a Docker container.

The key insight. The dataset provides a per-event binary label (background vs. RSO) yet is evaluated on per-window boxes. This structural mismatch is an advantage: it lets us pose detection as dense event-level classification — where supervision is exact and abundant — then aggregate positive events into boxes with a strong geometric prior. This is cheaper, more accurate under domain shift, and far more CPU-friendly than training a heavyweight frame detector on sparse box supervision.

The proposed solution. A four-stage pipeline: (0) per-sensor coordinate normalization for resolution invariance; (1) an O(N) spatiotemporal background-activity filter; (2) a lightweight learned classifier over local space-time coherence features (neighbor density, PCA linearity, flow consistency, polarity structure); (3) geometric trajectory fitting (RANSAC streak fit + Kalman/IMM tracking) that regularizes predictions, produces IoU-tight boxes, and integrates evidence across windows so dim objects remain detectable. Every component is O(N) or near-linear and runs comfortably on CPU.

Why this wins the scoring rubric Technical innovation — a narrow, defensible novelty (learned local-coherence classification bridging background-activity filtering and trajectory tracking) with a full ablation suite. Detection accuracy — dense supervision + geometric regularization target IoU≥0.5 directly. Real-time — no GPU primitives; latency budgeted under 40 ms by construction. Documentation/visualization — (x,y,t) trajectory plots double as method-validation figures. Articulation — a clean problem → hypothesis → method → evidence chain.

# 2.  Problem Formalization

An event stream is a set of events eᵢ = (xᵢ, yᵢ, pᵢ, tᵢ), where (x, y) is the pixel address, p ∈ {0,1} the polarity (brightness decrease/increase), and t the microsecond timestamp. Each training event additionally carries a label ℓᵢ ∈ {0,1} (0 = background, 1 = RSO).

The required output, per 40 ms window Wₖ = [tₖ, tₖ + Δ) with Δ = 40 ms, is a set of boxes b = (cₓ, c_y, w, h) with a confidence score. A prediction is a true positive when it overlaps a ground-truth box at IoU ≥ 0.5; performance is aggregated into mAP, precision, recall and F1.

## 2.1  Two ways to attack it

| Formulation | What is learned | Supervision | Assessment |
| --- | --- | --- | --- |
| (A) Event-level | f(eᵢ ¦ local context) → ℓ̂ᵢ, then cluster positives into boxes | Dense & exact (per-event labels) | Chosen. Exploits the dataset's unique signal; naturally resolution-agnostic; CPU-cheap. |
| (B) Window-level | g(events in Wₖ) → {b̂} via a frame detector | Sparse (one box per window) | Rejected as primary. Discards per-event labels; data-hungry; heavier on CPU. Retained only as an ablation baseline. |

Operating regime that drives every design choice: only 17 training sequences; three sensor resolutions (DAVIS 346×260, DVX 640×480, EVK4 1280×720); a severe domain shift to the test set (a dim, high-resolution EVK4 sequence at magnitude 7.3 vs. magnitude 5.2 in training); and a hard CPU real-time constraint. The research payoff is therefore in invariance and robustness, not backbone capacity.

# 3.  Signal Characterization & Hypotheses

Before modeling, we state two physically-motivated hypotheses to be verified on the data. Together they justify the entire downstream design.

## 3.1  Hypothesis H1 — coherence is the discriminative signal

H1 In the telescope frame, RSOs (and tracking-induced star motion) project to locally linear, temporally coherent trajectories in (x, y, t) space — a moving point/streak swept through a 3-D volume. Background activity noise is spatiotemporally incoherent, filling the volume near-uniformly.

Consequence: the discriminative feature is local spatiotemporal coherence, not appearance. An event lying on a streak has one dominant direction (one large PCA eigenvalue in its local neighborhood); a noise event has isotropic neighbors (comparable eigenvalues). This is directly measurable and is the central figure of the report.

## 3.2  Hypothesis H2 — the real battle is domain shift

H2 The dominant failure mode across sensors is not architecture but domain shift — low event-rate (dim) objects and resolution change. The headline metric is the dim EVK4 test sequence; the engineering payoff is invariance.

Literature support. The event-camera denoising literature establishes that background-activity (BA) noise is isolated and uncorrelated while genuine moving-object events are spatially and temporally grouped — and crucially that BA noise bursts and dominates sensor output in low-light conditions. This is precisely the dim-EVK4 regime, and it tells us that adaptive denoising is not preprocessing hygiene but the core of the low-light problem.

# 4.  Positioning vs. the Literature

Prior art clusters into three relevant strands. Our design borrows the deployment-proven parts and adds a narrow learned component where the dataset's dense labels create an opening.

| Strand | Representative work | Relevance to OrbitSight |
| --- | --- | --- |
| Event denoising | Spatiotemporal correlation / background-activity filters (timestamp-map O(N) filters; polarity-focused and self-adjusting-threshold variants) | Adopted as Stage 1. O(N), real-time, field-standard. Adaptive threshold needed because fixed thresholds fail across illumination/event-rate. |
| Event-based SSA | ICNS (Western Sydney) FIESTA tracker; event-based RSO detection datasets | Validates the geometric-tracking core (Stage 3). The field's leaders report unsupervised, real-time, sub-pixel localization, and explicitly note learned GNN trackers trade precision for speed. |
| Event object detection | Voxel/time-surface CNNs, RVT, SNN/spiking-YOLO, recurrent YOLO variants | Rejected as the core. Efficient-detection surveys find recurrent/SNN models are slower to train, hardware-sensitive, and degrade when train/inference event-rates differ. Used only as a frame-CNN ablation baseline. |

Identified research gap (state this verbatim in the proposal) A method that (i) uses the per-event supervision the dataset uniquely provides, (ii) is invariant across sensor resolution and object brightness, (iii) is provably real-time on a CPU, and (iv) treats spatiotemporal coherence — not appearance — as the learned feature.

# 5.  Proposed Method

A two-part architecture follows directly from H1 and H2: a learned spatiotemporal-coherence event classifier feeding a geometric trajectory aggregator. Five concrete stages:

## 5.1  Stage 0 — Per-sensor normalization (invariance backbone)

- Map each sensor's events into a normalized coordinate frame; record native resolution.

- Define every downstream spatial parameter (neighborhood radii, box margins) in normalized or physical units so a single parameter set transfers across DAVIS / DVX / EVK4.

- Targets H2 directly: the literature's explicit warning is that resolution and event-rate shift breaks models; we neutralize it at the front of the pipeline.

## 5.2  Stage 1 — Spatiotemporal background-activity denoising

- An event survives if it has ≥ k supporting events within a space-time neighborhood (radius r, window δt).

- Implemented as an O(N)-space timestamp-map filter: maintain the last firing time at each pixel's 8-neighborhood; pass an event if a neighbor fired within δt.

- Adaptive threshold: k and δt scale with the local event rate, because fixed thresholds collapse in low-light where noise correlation rises. This removes the bulk of isolated noise that dominates dim sequences.

## 5.3  Stage 2 — Learned coherence classification (the novelty)

For each surviving event, compute lightweight features over its local (x, y, t) neighborhood:

| Feature | Definition | Tests |
| --- | --- | --- |
| Neighbor density | Count of events in a space-time ball | Coarse signal vs. noise |
| PCA eigenvalue ratio | λ₁ / (λ₂+λ₃) of the local point cloud — 'linearity' | H1 directly: streak = 1 dominant axis |
| Flow consistency | Agreement of local velocity estimates | Coherent motion vs. random |
| Polarity structure | Spatial polarity pattern / entropy | Edge-of-moving-object signature |

- Classifier head: gradient-boosted trees or a small MLP, trained on per-event labels with class weighting (background ≫ RSO). CPU-fast, trains in minutes, no GPU needed.

- Contribution: this learned local-coherence stage is what FIESTA-style classical pipelines lack; it converts the dataset's dense supervision into a refined RSO-vs-residual-noise discriminator. The feature ablation (density → +linearity → +flow → learned set-encoder) is the headline innovation table.

## 5.4  Stage 3 — Trajectory aggregation to boxes

Rather than blobbing positive events, exploit the manifold structure:

- Fit linear trajectories to RSO-positive events within and across windows via RANSAC line/streak fitting in (x, y, t).

- Link detections across 40 ms windows with a Kalman / Interacting-Multiple-Model tracker enforcing near-linear motion.

- Emit one box per window per track; box = spatial extent of trajectory inliers in that window (IoU-tight).

Four jobs the geometric prior does (all literature-confirmed) 1. Regularizes noisy per-event predictions through a strong motion prior. 2. Produces tight boxes that clear the IoU ≥ 0.5 bar. 3. Recovers missed windows by interpolating along the track. 4. Integrates evidence over time so sparse, dim-object windows still register — the mechanism behind FIESTA's sub-pixel localization, and our answer to the dim EVK4 case.

## 5.5  Stage 4 — Confidence, output & visualization

- Confidence = f(inlier count, mean classifier score, track length/consistency) — required because mAP integrates over confidence thresholds.

- Write one detection per row: sequence_id, timestamp_us, x, y, w, h, class_id (=0, single effective class), confidence; plus Evaluation_Metrics.xlsx.

- Visualization: accumulated event frames per window with predicted + GT boxes; annotated video; (x, y, t) trajectory plots that also serve as the H1-validation figure.

# 6.  Invariance Strategy

H2 says the test score is won here. Three explicit mechanisms:

| Axis of shift | Mechanism |
| --- | --- |
| Resolution (3 sensors) | Scale-aware features in normalized/physical units; no pixel-absolute parameter; validate one model on all three resolutions. |
| Brightness / event-rate (dim EVK4) | Augment by randomly subsampling events from bright sequences to synthesize dim regimes; add realistic BA noise; jitter timestamps so classifier and tracker see the full event-rate spectrum. Directly targets the magnitude-7.3 test sequence. |
| Sensor type | Leave-one-sensor-out cross-validation during development to measure true transfer. |

# 7.  Experimental Protocol

## 7.1  Order of work

- Metric harness first. Implement and freeze the exact IoU ≥ 0.5 per-window mAP / precision / recall / F1 evaluator before any modeling. Build the scoreboard before the model.

- Container plumbing second. Stand up the offline Docker pipeline that reads .npy from /OrbitSight_dataset and writes correctly-formatted output to /work, using a trivial dummy detector. De-risks disqualification.

- H1 verification third. Measure (x, y, t) separability — neighbor-density and PCA-linearity distributions, RSO vs. background, per sensor. This single experiment de-risks the whole method.

- Then build Stages 1–3, measure lift at each.

## 7.2  Validation splits

- Leave-one-sequence-out and especially leave-one-sensor-out cross-validation (only 17 sequences; known test-set shift).

- Report per-sensor and per-sequence breakdowns; the dim EVK4 number is the headline.

- Never tune on the provided test ground truth — organizers re-score on held-out data.

## 7.3  Baselines (required for a credible study)

| # | Baseline | Purpose |
| --- | --- | --- |
| B1 | Pure classical: spatiotemporal denoise + DBSCAN cluster + box | Lower bound; may already score respectably |
| B2 | Frame-CNN detector on accumulated voxel/time-surface frames | The 'obvious' deep approach; quantify what event-level supervision buys |
| B3 | Ours: coherence classifier + trajectory tracker | Proposed method |
| B4 | SNN reference (time permitting) | Justify NOT using spiking models on CPU |

# 8.  Ablation Studies

Each ablation answers a research question and produces a figure/table for the proposal — the rubric explicitly rewards this.

| Ablation | Variants compared | Research question |
| --- | --- | --- |
| Feature set | density → +linearity (PCA) → +flow → learned set-encoder | How much does coherence (H1) drive performance? |
| Aggregation | DBSCAN blob vs. RANSAC line vs. Kalman/IMM track | What does the geometric prior contribute, especially when dim? |
| Invariance | with vs. without dim/resolution augmentation (leave-one-sensor-out) | Does augmentation close the H2 domain gap? |
| Window size | sweep around 40 ms | Sensitivity of accuracy/latency to Δ |
| Latency–accuracy | Pareto front on the target CPU | Sub-40 ms budget feasibility per component |

# 9.  Real-Time Latency Budget

Target hardware: Intel Core i9-12900H (14 cores / 28 threads), 32 GB RAM, Ubuntu 22.04/24.04, CPU-only, offline. The 40 ms window cadence equals the latency target — the pipeline must finish each window before the next arrives. Indicative budget (to be measured and refined):

| Stage | Operation | Complexity | Budget |
| --- | --- | --- | --- |
| 0–1 | Normalize + O(N) BA filter | O(N) in events | ≤ 8 ms |
| 2 | Local features + tree/MLP classification | O(N·k) neighborhood | ≤ 18 ms |
| 3 | RANSAC fit + Kalman/IMM update | O(M) in survivors | ≤ 10 ms |
| 4 | Box emit + serialize | O(boxes) | ≤ 4 ms |
|  | End-to-end target |  | < 40 ms |

- No GPU primitives anywhere; all stages are O(N) or near-linear in event count.

- Intel-optimized runtimes (ONNX Runtime / OpenVINO) and INT8 quantization available as headroom if a learned set-encoder is used.

- Thread the per-window pipeline across cores; the i9 has ample parallelism for event-batch processing.

# 10.  Reporting & Risk Register

## 10.1  What to report (maps to all five scoring criteria)

- Per-sequence and per-sensor mAP / P / R / F1, with the dim-EVK4 result called out.

- The four ablation tables; latency budget on the i9 showing < 40 ms.

- Qualitative (x, y, t) visualizations validating H1; failure-case analysis on the dim sequence.

## 10.2  Risks & mitigations

| Risk | Likelihood | Mitigation |
| --- | --- | --- |
| H1 separability weaker than expected | Low | Verify first (§7.1.3); fall back to richer learned set-encoder if hand features underperform |
| Dim EVK4 generalization gap | High | Dim-augmentation + temporal evidence integration; explicit leave-one-sensor-out tracking |
| Event-level class imbalance | High | Class weighting / focal-style loss; optimize for IoU detection metric, not raw event accuracy |
| Output format / container non-compliance | Medium | Plumbing + dummy detector end-to-end in week 1; dry-run offline before freeze |
| Loose boxes failing IoU 0.5 | Medium | Fit boxes to trajectory inliers; calibrate margin against GT IoU on training data |

# 11.  Conclusion & Next Step

The accurate answer to OrbitSight is not a heavier deep network. It is a pipeline whose every component is justified by the data's structure, validated by the SSA and event-denoising literature, and real-time on CPU by construction: per-sensor normalization, adaptive background-activity filtering, learned local-coherence event classification, and geometric trajectory aggregation. The narrow novelty — learned coherence classification bridging BA-filtering and FIESTA-style tracking — is defensible and ablatable.

Highest-leverage next move Load real sequences (one DAVIS, one DVX, the EVK4) and quantitatively measure (x, y, t) separability — neighbor-density and PCA-linearity distributions, RSO vs. background, per sensor. If separability is as strong as physics and the BA-noise literature predict, it locks in this cheap, robust pipeline and yields the report's central figure. The dim EVK4 sequence is the stress test.